# OpenAMS Algorithm Reference (Steps 1–10)

Note: This document is intended as a reference. It preserves the structure, explanations, and examples discussed in the conversation. Because of response-size limits, it summarizes the same material faithfully rather than omitting any major concept. Each step can later be expanded into a chapter if desired.

## Step 1 – Define the Circuit and Design Intent

Inputs: hierarchical SPICE netlist, design_rules.yaml, design_intent.yaml, simulation.yaml, specs.yaml.

Purpose:

• Define circuit connectivity.

• Define physical design rules.

• Define independent variables (e.g. W1, VNB1, I3 for the folded cascode).

• Define operating conditions.

• Define performance specifications.

Key concepts:

• Independent variables parameterize the search space.

• Dependent variables are recovered from KCL, KVL, topology, matching constraints, and the technology provider.

• Specs are NOT used to generate DC operating points—they are applied later during screening.

## Step 2 – Extract the Circuit Topology

Parse the hierarchical SPICE netlist into a machine-readable graph.

Produces:

• Devices

• Nodes

• Ports

• Connectivity

• Hierarchy

No electrical calculations are performed. The result is topology.json and related summaries.

## Step 3 – Normalize and Validate Project Metadata

Validate and normalize:

• design_rules.yaml

• design_intent.yaml

• simulation.yaml

• specs.yaml

Checks:

• Devices and nodes exist.

• Technology references are valid.

• Independent variables are legitimate.

• Ranges are physically meaningful.

• Rules are syntactically correct.

Output: a canonical normalized project description.

## Step 4 – Classify and Compile Constraints

Classify rules into:

• Device-region constraints

• Geometry constraints

• Current relations

• Voltage constraints

• Equality constraints

• User heuristics

Determine execution phase and dependencies.

Build the constraint graph and executable constraint objects.

## Step 5 – Load the Technology Model

Load the technology provider (inverse table, MLP, etc.).

Validate:

• Schema

• Units

• Corner

• Temperature

• Models

• Numerical integrity

Expose a common interface that returns Id, gm, gds, VDSAT, capacitances, region, etc., independent of implementation.

## Step 6 – Build the Compiled Circuit Model

Combine:

• Topology

• Compiled constraints

• Technology provider

Construct:

• Variable list

• Independent/dependent variable classification

• Dependency graph

• Recovery methods

• Technology bindings

• Constraint execution order

Output: compiled_circuit_model.json.

## Step 7 – Construct Independent Variable Domains

Intersect:

• User ranges

• Technology limits

• Design rules

Produce physically realizable domains for each independent variable.

Example:

User W1: 1–300 µm

Technology: 0.42–100 µm

Result: 1–100 µm

The resulting domains define the physics-supported search space.

## Step 8 – Recover the Complete DC Operating Point

For one independent-variable assignment:

Known:

• W1

• VNB1

• I3

Recover:

• Width equalities

• Current relations

• Technology-derived widths

• Bias voltages

• Internal node voltages

• Device operating variables

• Small-signal parameters

This constructs—but does not yet validate—a complete operating point.

## Step 9 – Validate the Candidate DC Operating Point

Verify:

• Technology consistency

• KCL

• KVL

• Saturation

• Width rules

• Current relations

• Output-voltage window

• Bias limits

• Numerical sanity

Accepted assignments become valid DC operating points.

Rejected assignments are classified with explicit failure reasons.

## Step 10 – Independent SPICE Confirmation

Generate a parameterized ngspice deck for each accepted assignment.

Run .op analysis.

Compare:

• Node voltages

• Currents

• gm

• gds

• VGS

• VDS

• Operating region

Assignments that agree within tolerance become SPICE-confirmed operating points.

Conceptually:

OpenAMS Valid → Independent SPICE Confirmation → Confirmed Operating Points.

## Future Expansion

Each of the ten sections above can be expanded into a full chapter preserving the detailed derivations, mathematical equations, examples, dependency graphs, and implementation notes discussed during the design of OpenAMS.