# OpenAMS Pre-Layout to Post-Layout Pipeline

Qualified two-stage ALIGN → Magic → Netgen LVS → full RCX flow

Operational reproducibility guide for producing netlist_post_layout.spice from a numerically sized pre-layout netlist.

# 1. Purpose

This document freezes the qualified OpenAMS two-stage post-layout flow developed during the ALIGN/Magic expedition. The user-facing contract is:

sized pre-layout netlist.spice
        ↓
physical witness selection
        ↓
ALIGN-compatible realization
        ↓
ALIGN placement + routing
        ↓
GDS
        ↓
Magic extraction
        ↓
Netgen LVS
        ↓
Magic full flat RCX
        ↓
netlist_post_layout.spice

# 2. Current qualified scope

The implementation is currently qualified for the seven-transistor two-stage op-amp topology. The orchestration pattern is intended to become topology-generic later.

M1 n1    inp   ntail vss
M2 n2    inn   ntail vss
M3 n1    n1    vdd   vdd
M4 n2    n1    vdd   vdd
M5 ntail vbias vss   vss
M6 out   n2    vdd   vdd
M7 out   vbias vss   vss

The qualified ALIGN SKY130 physical representation uses Wunit = 0.42 µm, L = 0.15 µm, with large effective width encoded primarily through legal even NF.

# 3. Required installations

OpenAMS:
~/AMS-Tutorial/openams
~/AMS-Tutorial/openams/.venv-openams

ALIGN:
~/AMS-Tutorial/ALIGN-public
~/AMS-Tutorial/ALIGN-public/.venv-align

ALIGN SKY130 PDK:
~/AMS-Tutorial/ALIGN-pdk-sky130/SKY130_PDK

Magic technology:
/usr/local/share/pdk/sky130A/libs.tech/magic/sky130A.tech

Netgen setup:
/usr/local/share/pdk/sky130A/libs.tech/netgen/sky130A_setup.tcl

The user remains in .venv-openams. The wrapper explicitly invokes ALIGN through ALIGN's own Python interpreter and schematic2layout.py.

# 4. Input contract

The input must be a numerically sized SPICE netlist. Unresolved placeholders are rejected. The qualified length is L=0.15 µm; a mismatched length is a hard failure.

XM1 ... L=0.15u W=1.0u ...
XM2 ... L=0.15u W=1.0u ...
XM3 ... L=0.15u W=5.0u ...
XM4 ... L=0.15u W=5.0u ...
XM5 ... L=0.15u W=5.0u ...
XM6 ... L=0.15u W=98.0u ...
XM7 ... L=0.15u W=50.0u ...

# 5. Physical witness source of truth

The physical realization pool is backend-neutral:

examples/two_stage_opamp/generated/assignment_synthesis/physical_witness_pool.csv

It provides physical_candidate_id, nf_m1...nf_m7, realized_w_m1_um...realized_w_m7_um, physical_legal, and width-error metadata. The wrapper selects a physical_legal row whose realized widths match the sized netlist.

# 6. ALIGN-specific realization

ALIGN then maps the target physical widths to Wunit=0.42 µm, L=0.15 µm, and the nearest legal even NF. This backend-specific translation is recorded in netlist_post_layout.align_realization.json.

| Device | Target W (µm) | ALIGN NF | ALIGN Wreal (µm) | Error |
| --- | --- | --- | --- | --- |
| M1 | 1.00 | 2 | 0.84 | 16.0% |
| M2 | 1.00 | 2 | 0.84 | 16.0% |
| M3 | 5.00 | 12 | 5.04 | 0.8% |
| M4 | 5.00 | 12 | 5.04 | 0.8% |
| M5 | 5.00 | 12 | 5.04 | 0.8% |
| M6 | 98.0 | 234 | 98.28 | 0.286% |
| M7 | 50.0 | 120 | 50.40 | 0.8% |

The 16% minimum-device discrepancy on M1/M2 is currently accepted for the physical-flow proof and must be evaluated in post-layout performance simulation.

# 7. Exact one-command execution

cd ~/AMS-Tutorial/openams
source .venv-openams/bin/activate

rm -rf runtime/postlayout/two_stage

python tools/postlayout/openams_postlayout.py \
  examples/two_stage_opamp/generated/sized_netlist.spice \
  --output-netlist netlist_post_layout.spice \
  --run-dir runtime/postlayout/two_stage

# 8. Pipeline stages and acceptance criteria

## Stage 1 - Environment qualification

Checks input files, physical witness pool, Magic, Netgen, ngspice, ALIGN Python, and ALIGN CLI.

## Stage 2 - Canonical sized-netlist validation

Requires .subckt, numeric M1-M7 W/L, no unresolved placeholders, and L=0.15 µm.

## Stage 3 - Physical witness selection

Matches the sized netlist against physical_witness_pool.csv and writes 02_physical_mapping/mapping.json.

## Stage 4A - ALIGN input generation

Creates an ALIGN-compatible netlist using W=0.42 µm, L=0.15 µm, and legal even NF.

## Stage 4B - ALIGN P&R

Runs ALIGN and requires OPENAMS_TWO_STAGE_0.gds plus an empty ALIGN error file.

## Stage 4C - Magic extraction

Imports GDS and uses readspice to restore top-level port order.

## Stage 4D - Netgen LVS

Uses finger-expanded reference and D/S permutation. PASS requires 'Circuits match uniquely'.

## Stage 4E - Full flat RCX

Flattens, enables resistance extraction, forces extresist all, and outputs full RCX SPICE.

## Stage 5 - Artifact validation

Requires LVS evidence and nonzero R, C, and X counts before installing netlist_post_layout.spice.

# 9. Proven Magic/Netgen commands

Top-level port restoration:

gds read <ALIGN_GDS>
load OPENAMS_TWO_STAGE_0
readspice <reference_ports.spice>
extract all
ext2spice lvs
ext2spice subcircuit on

LVS source/drain normalization:

permute pins sky130_fd_pr__nfet_01v8_lvt D S
permute pins sky130_fd_pr__pfet_01v8 D S

Full analog RCX:

flatten OPENAMS_TWO_STAGE_FLAT
load OPENAMS_TWO_STAGE_FLAT

extract do unique
extract do resistance
extresist all
extresist extout on
extract all

ext2spice lvs
ext2spice subcircuit on
ext2spice cthresh 0
ext2spice extresist on

# 10. Generated artifacts

runtime/postlayout/two_stage/
├── summary.json
├── physical_backend.log
├── 02_physical_mapping/
│   └── mapping.json
├── 03_layout/
│   ├── align/
│   └── align_input/
└── 04_pex/
    └── magic/
        ├── align_realization.json
        ├── lvs_report.out
        ├── openams_two_stage_with_ports.spice
        ├── openams_two_stage_xref.spice
        └── openams_two_stage_rcx_force_all.spice

repository root:
├── netlist_post_layout.spice
└── netlist_post_layout.align_realization.json

# 11. Known non-blocking warnings

Magic currently reports unknown ALIGN GDS layer/datatype warnings for several marker layers (for example layer 100/type 5, 235/type 5, and 104/type 0). In qualified runs these did not prevent SKY130 MOS extraction, unique LVS match, or full RCX. Netgen also reports model-stub property warnings; the acceptance criterion remains the final unique circuit match.

# 12. Current completion boundary

The one-command implementation currently certifies physical implementation through LVS-clean full RCX and creation of netlist_post_layout.spice. The next qualification stage is post-layout ngspice .op and AC performance comparison using the same testbench as pre-layout.

netlist_post_layout.spice
        ↓
same pre-layout testbench
        ↓
ngspice .op
        ↓
ngspice AC
        ↓
gain / UGB / phase margin / VOUT / power
        ↓
pre-layout vs post-layout delta

# 13. Installation

cd ~/AMS-Tutorial/openams

cp ~/Downloads/openams_postlayout_v5.py \
  tools/postlayout/openams_postlayout.py

cp ~/Downloads/run_align_magic_two_stage_dynamic.sh \
  tools/postlayout/run_align_magic_two_stage_dynamic.sh

chmod +x \
  tools/postlayout/openams_postlayout.py \
  tools/postlayout/run_align_magic_two_stage_dynamic.sh

Recommended documentation location: docs/POST_LAYOUT_PIPELINE.md

# Appendix A - openams_postlayout.py

#!/usr/bin/env python3
from __future__ import annotations

import argparse
import csv
import json
import re
import shutil
import subprocess
import sys
from pathlib import Path


class PipelineError(RuntimeError):
    pass


def fail(msg: str):
    raise PipelineError(msg)


def spice_si(x: str) -> float:
    x = x.strip().lower()
    mult = {"t":1e12,"g":1e9,"meg":1e6,"k":1e3,"m":1e-3,
            "u":1e-6,"n":1e-9,"p":1e-12,"f":1e-15}
    m = re.fullmatch(r"([+-]?(?:\d+(?:\.\d*)?|\.\d+)(?:e[+-]?\d+)?)(meg|[tgkmunpf])?", x)
    if not m:
        fail(f"cannot parse SPICE value: {x}")
    return float(m.group(1)) * mult.get(m.group(2), 1.0)


def logical_lines(text: str):
    out, cur = [], ""
    for raw in text.splitlines():
        s = raw.strip()
        if not s or s.startswith("*"):
            continue
        if s.startswith("+"):
            cur += " " + s[1:].strip()
        else:
            if cur:
                out.append(cur)
            cur = s
    if cur:
        out.append(cur)
    return out


def parse_netlist(path: Path):
    text = path.read_text(errors="replace")
    ph = sorted(set(re.findall(r"\{[^{}]+\}", text)))
    if ph:
        fail("input netlist still has unresolved placeholders: " + ", ".join(ph[:10]))

    subckt = None
    pins = []
    geom = {}
    for line in logical_lines(text):
        t = line.split()
        if not t:
            continue
        if t[0].lower() == ".subckt" and subckt is None:
            subckt, pins = t[1], t[2:]
        m = re.fullmatch(r"X?M(\d+)", t[0].upper())
        if not m:
            continue
        params = {}
        for tok in t:
            if "=" in tok:
                k,v = tok.split("=",1)
                params[k.lower()] = v
        if "w" in params and "l" in params:
            geom[f"M{int(m.group(1))}"] = {
                "w_um": spice_si(params["w"]) * 1e6,
                "l_um": spice_si(params["l"]) * 1e6,
            }
    if subckt is None:
        fail("no .subckt found")
    if len(geom) != 7:
        fail(f"expected M1..M7 with numeric W/L; found {sorted(geom)}")
    return {"subckt": subckt, "pins": pins, "geom": geom}


def load_physical_witness(info, pool: Path, tol_um=0.11):
    rows = list(csv.DictReader(pool.open(newline="")))
    candidates = []
    for row in rows:
        if str(row.get("physical_legal","")).lower() not in ("true","1","yes"):
            continue
        err = 0.0
        ok = True
        for i in range(1,8):
            key = f"realized_w_m{i}_um"
            if not row.get(key):
                ok = False
                break
            e = abs(info["geom"][f"M{i}"]["w_um"] - float(row[key]))
            if e > tol_um:
                ok = False
                break
            err += e
        if ok:
            candidates.append((err,row))
    if not candidates:
        fail(f"no physical witness matches the sized netlist within {tol_um}um")
    candidates.sort(key=lambda x:x[0])
    row = candidates[0][1]
    devices = {}
    for i in range(1,8):
        devices[f"M{i}"] = {
            "requested_w_um": info["geom"][f"M{i}"]["w_um"],
            "requested_l_um": info["geom"][f"M{i}"]["l_um"],
            "nf": int(float(row[f"nf_m{i}"])),
            "realized_w_um": float(row[f"realized_w_m{i}_um"]),
            "width_error_um": float(row.get(f"width_error_m{i}_um") or 0),
            "width_rel_error": float(row.get(f"width_rel_error_m{i}") or 0),
        }
    return {
        "physical_candidate_id": row.get("physical_candidate_id"),
        "source_candidate_id": row.get("source_candidate_id"),
        "physical_legal": True,
        "vbias_v": float(row["vbias_v"]) if row.get("vbias_v") else None,
        "vout_v": float(row["vout_v"]) if row.get("vout_v") else None,
        "devices": devices,
    }


def main():
    ap = argparse.ArgumentParser()
    ap.add_argument("netlist", type=Path)
    ap.add_argument("--output-netlist", type=Path, default=Path("netlist_post_layout.spice"))
    ap.add_argument("--run-dir", type=Path, default=Path("runtime/postlayout/latest"))
    ap.add_argument("--root", type=Path, default=Path("."))
    ap.add_argument(
        "--physical-witness-pool", type=Path,
        default=Path("examples/two_stage_opamp/generated/assignment_synthesis/physical_witness_pool.csv")
    )
    ap.add_argument(
        "--backend-script", type=Path,
        default=Path("tools/postlayout/run_align_magic_two_stage_dynamic.sh")
    )
    ap.add_argument("--align-length-um", type=float, default=0.15)
    args = ap.parse_args()

    root = args.root.resolve()
    netlist = args.netlist if args.netlist.is_absolute() else root / args.netlist
    pool = args.physical_witness_pool if args.physical_witness_pool.is_absolute() else root / args.physical_witness_pool
    backend = args.backend_script if args.backend_script.is_absolute() else root / args.backend_script
    run_dir = args.run_dir if args.run_dir.is_absolute() else root / args.run_dir
    output = args.output_netlist if args.output_netlist.is_absolute() else root / args.output_netlist
    run_dir.mkdir(parents=True, exist_ok=True)

    summary = {"status":"RUNNING","input":str(netlist),"output":str(output),"stages":{}}
    def save():
        (run_dir/"summary.json").write_text(json.dumps(summary,indent=2,sort_keys=True)+"\n")

    try:
        print("[1/6] Environment")
        for cmd in ("magic","netgen","ngspice"):
            if shutil.which(cmd) is None:
                fail(f"required command not found: {cmd}")
        for p,label in ((netlist,"input netlist"),(pool,"physical witness pool"),(backend,"ALIGN backend")):
            if not p.is_file():
                fail(f"{label} not found: {p}")
        align_py = Path.home()/"AMS-Tutorial/ALIGN-public/.venv-align/bin/python"
        align_cli = Path.home()/"AMS-Tutorial/ALIGN-public/.venv-align/bin/schematic2layout.py"
        if not align_py.is_file() or not align_cli.is_file():
            fail("ALIGN virtual environment/CLI n

ot found")
        summary["stages"]["environment"]="PASS"
        save(); print("PASS")

        print("\n[2/6] Canonical sized netlist")
        info = parse_netlist(netlist)
        lengths = {round(d["l_um"],9) for d in info["geom"].values()}
        if lengths != {round(args.align_length_um,9)}:
            fail(
                "input netlist length is incompatible with the qualified ALIGN technology flow: "
                f"found {sorted(lengths)} um, expected {args.align_length_um:g} um"
            )
        summary["canonical"]=info
        summary["stages"]["canonical"]="PASS"
        save(); print(f"PASS: {info['subckt']} M1..M7 L={args.align_length_um:g}um")

        print("\n[3/6] Physical witness")
        mapping = load_physical_witness(info,pool)
        mdir = run_dir/"02_physical_mapping"
        mdir.mkdir(parents=True,exist_ok=True)
        mapping_json = mdir/"mapping.json"
        mapping_json.write_text(json.dumps(mapping,indent=2,sort_keys=True)+"\n")
        summary["physical_mapping"]=mapping
        summary["stages"]["physical_witness"]="PASS"
        save()
        print("PASS:",mapping["physical_candidate_id"])
        for n,d in mapping["devices"].items():
            print(f"  {n}: nf={d['nf']} Wreal={d['realized_w_um']:g}um")

        print("\n[4/6] ALIGN -> Magic -> LVS -> RCX")
        env = dict(**__import__("os").environ)
        env.update({
            "OPENAMS_ROOT":str(root),
            "OPENAMS_PHYSICAL_MAPPING":str(mapping_json),
            "OPENAMS_OUTPUT_NETLIST":str(output),
            "ALIGN_WORK":str(run_dir/"03_layout/align"),
            "MAGIC_WORK":str(run_dir/"04_pex/magic"),
            "ALIGN_INPUT_WORK":str(run_dir/"03_layout/align_input"),
            "ALIGN_PYTHON":str(align_py),
            "ALIGN_CLI":str(align_cli),
        })
        log = run_dir/"physical_backend.log"
        with log.open("w") as f:
            p = subprocess.run(["bash",str(backend)],cwd=root,env=env,stdout=f,stderr=subprocess.STDOUT)
        if p.returncode:
            tail = "\n".join(log.read_text(errors="replace").splitlines()[-80:])
            fail(f"physical backend failed (rc={p.returncode})\n{tail}")
        if not output.is_file():
            fail("backend completed without post-layout netlist")
        summary["stages"]["physical_backend"]="PASS"
        summary["backend_log"]=str(log)
        save(); print("PASS")

        print("\n[5/6] Post-layout artifact validation")
        text = output.read_text(errors="replace")
        counts = {
            "R":sum(1 for l in text.splitlines() if l.startswith("R")),
            "C":sum(1 for l in text.splitlines() if l.startswith("C")),
            "X":sum(1 for l in text.splitlines() if l.startswith("X")),
        }
        lvs = run_dir/"04_pex/magic/lvs_report.out"
        if not lvs.is_file() or "Circuits match uniquely" not in lvs.read_text(errors="replace"):
            fail("LVS PASS evidence missing")
        if min(counts["R"],counts["C"],counts["X"]) <= 0:
            fail(f"incomplete RCX counts: {counts}")
        summary["rcx_counts"]=counts
        summary["stages"]["artifact_validation"]="PASS"
        save()
        print("PASS:",counts)

        print("\n[6/6] Result")
        summary["status"]="PASS_POST_LAYOUT_NETLIST_CREATED"
        save()
        print("PASS:",output)
        print("summary:",run_dir/"summary.json")
        print("NOTE: ngspice .op/AC validation is the next stage; this command certifies ALIGN/LVS/RCX output.")

        return 0

    except PipelineError as e:
        summary["status"]="FAIL"
        summary["error"]=str(e)
        save()
        print("\nRESULT: FAIL",file=sys.stderr)
        print(e,file=sys.stderr)
        print("summary:",run_dir/"summary.json",file=sys.stderr)
        return 2


if __name__ == "__main__":
    raise SystemExit(main())

# Appendix B - run_align_magic_two_stage_dynamic.sh

#!/usr/bin/env bash
set -euo pipefail

# Dynamic OpenAMS two-stage ALIGN -> Magic -> LVS -> full RCX backend.
#
# Required from openams_postlayout.py:
#   OPENAMS_ROOT
#   OPENAMS_PHYSICAL_MAPPING
#   OPENAMS_OUTPUT_NETLIST
#
# Optional:
#   ALIGN_PDK_ROOT
#   ALIGN_PYTHON
#   ALIGN_CLI
#   ALIGN_WORK
#   MAGIC_WORK
#   MAGIC_TECH
#   NETGEN_SETUP
#   ALIGN_UNIT_W_UM
#   ALIGN_L_UM
#   ALIGN_MAX_WIDTH_REL_ERROR

OPENAMS_ROOT="${OPENAMS_ROOT:-$HOME/AMS-Tutorial/openams}"
MAPPING="${OPENAMS_PHYSICAL_MAPPING:?OPENAMS_PHYSICAL_MAPPING is required}"
OUTPUT="${OPENAMS_OUTPUT_NETLIST:-$OPENAMS_ROOT/netlist_post_layout.spice}"

ALIGN_PDK_ROOT="${ALIGN_PDK_ROOT:-$HOME/AMS-Tutorial/ALIGN-pdk-sky130}"
ALIGN_PYTHON="${ALIGN_PYTHON:-$HOME/AMS-Tutorial/ALIGN-public/.venv-align/bin/python}"
ALIGN_CLI="${ALIGN_CLI:-$HOME/AMS-Tutorial/ALIGN-public/.venv-align/bin/schematic2layout.py}"

ALIGN_WORK="${ALIGN_WORK:-/tmp/openams_two_stage_align}"
MAGIC_WORK="${MAGIC_WORK:-/tmp/openams_two_stage_magic}"
ALIGN_INPUT_WORK="${ALIGN_INPUT_WORK:-$MAGIC_WORK/align_input}"

MAGIC_TECH="${MAGIC_TECH:-/usr/local/share/pdk/sky130A/libs.tech/magic/sky130A.tech}"
NETGEN_SETUP="${NETGEN_SETUP:-/usr/local/share/pdk/sky130A/libs.tech/netgen/sky130A_setup.tcl}"

ALIGN_UNIT_W_UM="${ALIGN_UNIT_W_UM:-0.42}"
ALIGN_L_UM="${ALIGN_L_UM:-0.15}"
ALIGN_MAX_WIDTH_REL_ERROR="${ALIGN_MAX_WIDTH_REL_ERROR:-0.20}"

need() {
    command -v "$1" >/dev/null 2>&1 || {
        echo "ERROR: required command not found: $1" >&2
        exit 1
    }
}

need magic
need netgen
need python

[[ -x "$ALIGN_PYTHON" ]] || { echo "ERROR: ALIGN Python not found: $ALIGN_PYTHON" >&2; exit 1; }
[[ -f "$ALIGN_CLI" ]] || { echo "ERROR: ALIGN CLI not found: $ALIGN_CLI" >&2; exit 1; }
[[ -f "$MAPPING" ]] || { echo "ERROR: physical mapping not found: $MAPPING" >&2; exit 1; }
[[ -f "$MAGIC_TECH" ]] || { echo "ERROR: Magic tech not found: $MAGIC_TECH" >&2; exit 1; }
[[ -f "$NETGEN_SETUP" ]] || { echo "ERROR: Netgen setup not found: $NETGEN_SETUP" >&2; exit 1; }

CONST_SRC="$ALIGN_PDK_ROOT/examples/openams_two_stage/openams_two_stage.const.json"
[[ -f "$CONST_SRC" ]] || { echo "ERROR: ALIGN constraint file not found: $CONST_SRC" >&2; exit 1; }

rm -rf "$ALIGN_WORK" "$MAGIC_WORK"
mkdir -p "$ALIGN_WORK" "$MAGIC_WORK" "$ALIGN_INPUT_WORK" "$(dirname "$OUTPUT")"

echo "============================================================"
echo " OPENAMS DYNAMIC ALIGN -> MAGIC -> LVS -> RCX"
echo "============================================================"

echo
echo "[A1] Build ALIGN-compatible circuit from physical witness"

python - "$MAPPING" "$ALIGN_INPUT_WORK/openams_two_stage.sp" \
         "$MAGIC_WORK/align_realization.json" \
         "$MAGIC_WORK/openams_two_stage_xref.spice" \
         "$ALIGN_UNIT_W_UM" "$ALIGN_L_UM" "$ALIGN_MAX_WIDTH_REL_ERROR" <<'PY'
import json, math, sys
from pathlib import Path

mapping_path, spice_out, manifest_out, xref_out = map(Path, sys.argv[1:5])
unit_w = float(sys.argv[5])
align_l = float(sys.argv[6])
max_rel = float(sys.argv[7])

m = json.loads(mapping_path.read_text())
devs = m["devices"]

def even_nf_for(target_w):
    raw = target_w / unit_w
    nf = max(2, int(round(raw / 2.0)) * 2)
    return nf

align = {}
for name, d in devs.items():
    target = float(d["realized_w_um"])
    nf = even_nf_for(target)
    realized = unit_w * nf
    rel = abs(realized - target) / max(abs(target), 1e-30)
    if rel > max_rel:
        raise SystemExit(
            f"ERROR: {name}: ALIGN mapping error {100*rel:.2f}% exceeds "
            f"{100*max_rel:.2f}% (target={target}um, Wunit={unit_w}um, NF={nf})"
        )
    align[name] = {
        "target_w_um": target,
        "align_unit_w_um": unit_w,
        "align_l_um": align_l,
        "align_nf": nf,
        "align_realized_w_um": realized,
        "align_width_rel_error": rel,
    }

# Fixed two-stage topology; sizes come entirely from the selected physical witness.
sp = [
    ".subckt openams_two_stage inp inn out vdd vss vbias",
    "",
]
topo = {
    "M1": ("n1", "inp", "ntail", "vss", "sky130_fd_pr__nfet_01v8_lvt"),
    "M2": ("n2", "inn", "ntail", "vss", "sky130_fd_pr__nfet_01v8_lvt"),
    "M3": ("n1", "n1", "vdd", "vdd", "sky130_fd_pr__pfet_01v8"),
    "M4": ("n2", "n1", "vdd", "vdd", "sky130_fd_pr__pfet_01v8"),
    "M5": ("ntail", "vbias", "vss", "vss", "sky130_fd_pr__nfet_01v8_lvt"),
    "M6": ("out", "n2", "vdd", "vdd", "sky130_fd_pr__pfet_01v8"),
    "M7": ("out", "vbias", "vss", "vss", "sky130_fd_pr__nfet_01v8_lvt"),
}
for name in [f"M{i}" for i in range(1,8)]:
    d,g,s,b,model = topo[name]
    a = align[name]
    sp.append(
        f"{name} {d} {g} {s} {b} {model} "
        f"L={align_l*1e-6:.12g} W={unit_w*1e-6:.12g} "
        f"NF={a['align_nf']} M=1 STACK=1"
    )
sp += ["", ".ends openams_two_stage", ""]
spice_out.write_text("\n".join(sp))

manifest = {
    "physical_candidate_id": m.get("physical_candidate_id"),
    "source_physical_mapping": str(mapping_path),
    "align_unit_w_um": unit_w,
    "align_l_um": align_l,
    "devices": align,
}
manifest_out.write_text(json.dumps(manifest, indent=2, sort_keys=True) + "\n")

# Exact finger-expanded LVS reference corresponding to the ALIGN input.
lines = [
    ".subckt sky130_fd_pr__nfet_01v8_lvt D G S B",
    ".ends sky130_fd_pr__nfet_01v8_lvt",
    "",
    ".subckt sky130_fd_pr__pfet_01v8 D G S B",
    ".ends sky130_fd_pr__pfet_01v8",
    "",
    ".subckt OPENAMS_TWO_STAGE_0 INP INN OUT VDD VSS VBIAS",
]
def add(name, count, d, g, s, b, model):
    for i in range(count):
        lines.append(
            f"X{name}_{i} {d} {g} {s} {b} {model} "
            f"w={unit_w:g} l={align_l:g}"
        )
for name in [f"M{i}" for i in range(1,8)]:
    d,g,s,b,model = topo[name]
    add(name, align[name]["align_nf"], d.upper(), g.upper(), s.upper(), b.upper(), model)
lines.append(".ends OPENAMS_TWO_STAGE_0")
xref_out.write_text("\n".join(lines) + "\n")

print(f"physical_candidate_id: {m.get('physical_candidate_id')}")
for name in [f"M{i}" for i in r

ange(1,8)]:
    a = align[name]
    print(
        f"  {name}: target={a['target_w_um']:g}um -> "
        f"ALIGN W={unit_w:g}um NF={a['align_nf']} -> "
        f"{a['align_realized_w_um']:g}um "
        f"(err={100*a['align_width_rel_error']:.2f}%)"
    )
print("total ALIGN fingers:", sum(a["align_nf"] for a in align.values()))
PY

cp "$CONST_SRC" "$ALIGN_INPUT_WORK/openams_two_stage.const.json"

echo
echo "[A2] ALIGN placement and routing"
(
    cd "$ALIGN_PDK_ROOT"
    "$ALIGN_PYTHON" "$ALIGN_CLI" \
        "$ALIGN_INPUT_WORK" \
        -p SKY130_PDK \
        -w "$ALIGN_WORK"
)

ALIGN_GDS="$ALIGN_WORK/OPENAMS_TWO_STAGE_0.gds"
ALIGN_ERRORS="$ALIGN_WORK/3_pnr/OPENAMS_TWO_STAGE_0.errors"
[[ -s "$ALIGN_GDS" ]] || { echo "ERROR: ALIGN GDS missing: $ALIGN_GDS" >&2; exit 1; }
if [[ -f "$ALIGN_ERRORS" && -s "$ALIGN_ERRORS" ]]; then
    echo "ERROR: ALIGN reported errors:" >&2
    cat "$ALIGN_ERRORS" >&2
    exit 1
fi
echo "PASS: ALIGN GDS $ALIGN_GDS"

echo
echo "[A3] Magic extraction + top-level ports"
cat > "$MAGIC_WORK/reference_ports.spice" <<'EOF'
.subckt OPENAMS_TWO_STAGE_0 INP INN OUT VDD VSS VBIAS
.ends OPENAMS_TWO_STAGE_0
EOF

cat > "$MAGIC_WORK/extract_lvs.tcl" <<EOF
gds read $ALIGN_GDS
load OPENAMS_TWO_STAGE_0
readspice $MAGIC_WORK/reference_ports.spice
extract all
ext2spice lvs
ext2spice subcircuit on
ext2spice -o $MAGIC_WORK/openams_two_stage_with_ports.spice
quit -noprompt
EOF

(
    cd "$MAGIC_WORK"
    magic -dnull -noconsole -T "$MAGIC_TECH" extract_lvs.tcl
)

grep -q '^\.subckt OPENAMS_TWO_STAGE_0 INP INN OUT VDD VSS VBIAS' \
    "$MAGIC_WORK/openams_two_stage_with_ports.spice" || {
    echo "ERROR: Magic top-level ports are incorrect" >&2
    exit 1
}

echo
echo "[A4] Netgen LVS"
cat > "$MAGIC_WORK/model_stubs.spice" <<'EOF'
.subckt sky130_fd_pr__nfet_01v8_lvt D G S B
.ends sky130_fd_pr__nfet_01v8_lvt
.subckt sky130_fd_pr__pfet_01v8 D G S B
.ends sky130_fd_pr__pfet_01v8
EOF

cat "$MAGIC_WORK/model_stubs.spice" \
    "$MAGIC_WORK/openams_two_stage_with_ports.spice" \
    > "$MAGIC_WORK/openams_two_stage_layout_lvs.spice"

cp "$NETGEN_SETUP" "$MAGIC_WORK/sky130A_openams_setup.tcl"
cat >> "$MAGIC_WORK/sky130A_openams_setup.tcl" <<'EOF'
permute pins sky130_fd_pr__nfet_01v8_lvt D S
permute pins sky130_fd_pr__pfet_01v8 D S
EOF

(
    cd "$MAGIC_WORK"
    netgen -batch lvs \
        "openams_two_stage_layout_lvs.spice OPENAMS_TWO_STAGE_0" \
        "openams_two_stage_xref.spice OPENAMS_TWO_STAGE_0" \
        sky130A_openams_setup.tcl \
        lvs_report.out
)
grep -q 'Circuits match uniquely' "$MAGIC_WORK/lvs_report.out" || {
    echo "ERROR: LVS did not match uniquely" >&2
    tail -100 "$MAGIC_WORK/lvs_report.out" >&2
    exit 1
}
echo "PASS: LVS circuits match uniquely"

echo
echo "[A5] Full flat Magic RCX"
cat > "$MAGIC_WORK/extract_rcx_force_all.tcl" <<EOF
gds read $ALIGN_GDS
load OPENAMS_TWO_STAGE_0
readspice $MAGIC_WORK/reference_ports.spice
flatten OPENAMS_TWO_STAGE_FLAT
load OPENAMS_TWO_STAGE_FLAT
extract do unique
extract do resistance
extresist all
extresist extout on
extract all
ext2spice lvs
ext2spice subcircuit on
ext2spice cthresh 0
ext2spice extresist on
ext2spice -o $MAGIC_WORK/openams_two_stage_rcx_force_all.spice
quit -noprompt
EOF

(
    cd "$MAGIC_WORK"
    rm -f OPENAMS_TWO_STAGE_FLAT.ext OPENAMS_TWO_STAGE_FLAT.res.ext
    magic -dnull -noconsole -T "$MAGIC_TECH" extract_rcx_force_all.tcl
)

RCX="$MAGIC_WORK/openams_two_stage_rcx_force_all.spice"
[[ -s "$RCX" ]] || { echo "ERROR: RCX netlist missing" >&2; exit 1; }

R_COUNT="$(grep -c '^R' "$RCX" || true)"
C_COUNT="$(grep -c '^C' "$RCX" || true)"
X_COUNT="$(grep -c '^X' "$RCX" || true)"
echo "PASS: RCX R=$R_COUNT C=$C_COUNT X=$X_COUNT"

cp "$RCX" "$OUTPUT"
cp "$MAGIC_WORK/align_realization.json" "${OUTPUT%.spice}.align_realization.json"

echo
echo "============================================================"
echo " DYNAMIC ALIGN PHYSICAL FLOW PASS"
echo "============================================================"
echo "Output: $OUTPUT"
echo "ALIGN realization: ${OUTPUT%.spice}.align_realization.json"