# OpenAMS Generic Hierarchical Pipeline Runbook

End-to-end execution and stage-by-stage restart commands

Purpose. This runbook documents the current topology-agnostic hierarchical flow used to build component feasibility datasets, train component MLPs, and generate full-circuit valid witnesses. The folded-cascode example is shown concretely, while each stage also includes a generic template that can be reused for another topology.

Important architecture note. The orchestration, dataset generation, MLP training, and hierarchical witness engine are generic. A compiled contract may still reference a topology-specific exact-realizer adapter. The commands below do not hard-code folded-cascode component names, transistor numbers, or model feature names.

# Quick Start - The Three Canonical Commands

Summary. The current generic OpenAMS pre-layout flow is split into three stable command boundaries. Command 1 starts from topology metadata and produces the complete valid-witness set. Command 2 estimates pre-SPICE metrics and selects a compact, diverse SPICE candidate set. Command 3 runs ngspice on every selected candidate in parallel and merges the measured results.

| Command | Entry point | Primary input | Primary output |
| --- | --- | --- | --- |
| 1 | run_generic_hierarchical_pipeline.py | design_intent.yaml + referenced topology metadata | valid_witnesses.csv |
| 2 | run_pre_spice_candidate_pipeline.py | valid_witnesses.csv + compiled contract | ngspice_candidates_<N>.csv |
| 3 | run_ngspice_candidates_parallel.py | selected candidates + ngspice_validation.yaml | validated_candidates.csv |

Generic three-command template:

# 1 - topology metadata -> full exact valid-witness set
python tools/run_generic_hierarchical_pipeline.py \
  --intent examples/<topology>/inputs/design_intent.yaml \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --clean

# 2 - valid witnesses -> estimated metrics -> selected SPICE candidates
python tools/run_pre_spice_candidate_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --technology-csv technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output examples/<topology>/generated/assignment_synthesis/pre_spice_metrics.csv \
  --candidates-output examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

# 3 - selected candidates -> parallel ngspice -> measured/validated candidates
python tools/validation/run_ngspice_candidates_parallel.py \
  --plan examples/<topology>/inputs/ngspice_validation.yaml \
  --candidates examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --root "$PWD" \
  --workers 12 \
  --output-dir runtime/ngspice_<topology>_selected_100 \
  --overwrite

Regression-qualified two-stage op-amp example:

# 1) Full hierarchical witness generation
python tools/run_generic_hierarchical_pipeline.py \
  --intent examples/two_stage_opamp/inputs/design_intent.yaml \
  --contract examples/two_stage_opamp/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/two_stage_pipeline_L015 \
  --output examples/two_stage_opamp/generated/assignment_synthesis/two_stage_valid_witnesses_L015.csv \
  --workers 12 \
  --clean

# 2) Pre-SPICE estimation and 100-candidate selection
python tools/run_pre_spice_candidate_pipeline.py \
  --contract examples/two_stage_opamp/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses examples/two_stage_opamp/generated/assignment_synthesis/two_stage_valid_witnesses_L015.csv \
  --technology-csv technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output examples/two_stage_opamp/generated/assignment_synthesis/two_stage_pre_spice_metrics_L015.csv \
  --candidates-output examples/two_stage_opamp/generated/assignment_synthesis/two_stage_ngspice_candidates_100_regression.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

# 3) Parallel ngspice validation of exactly those 100 selected candidates
python tools/validation/run_ngspice_candidates_parallel.py \
  --plan examples/two_stage_opamp/inputs/ngspice_validation.yaml \
  --candidates examples/two_stage_opamp/generated/assignment_synthesis/two_stage_ngspice_candidates_100_regression.csv \
  --root "$PWD" \
  --workers 12 \
  --output-dir runtime/ngspice_two_stage_selected_100_regression \
  --overwrite

Two-stage regression reference. The latest generic runner reproduced 54 DC/total PASS rows out of 100, with maximum AC gain about 33.66 dB and median AC gain about 32.58 dB.

# Required Metadata Structures for the Entire Pipeline

Design principle. Topology-specific knowledge belongs in input metadata, the SPICE source, and compiled contracts. Generic Python tools should not contain topology names, fixed transistor counts, or circuit-specific equations.

| Artifact | Authored/generated | Consumed by | Required responsibility |
| --- | --- | --- | --- |
| SPICE netlist | Authored | exact realizers + ngspice | Topology, device models, parameter placeholders, canonical W/L convention |
| design_rules.yaml | Authored | compiler / realization flow | Technology and design-rule constraints |
| design_intent.yaml | Authored | contract compilers | Hierarchy, component model/training metadata, interfaces, exact realization, pre_spice_metrics |
| simulation.yaml | Authored where used | compiler / simulation flow | Environment, analyses, supplies, loads, tolerances |
| hierarchical_component_contract.json | Generated | datasets + training + witness engine + metric estimator | Resolved execution contract and preserved metric metadata |
| component .pt checkpoint | Generated | hierarchical witness engine | Features, normalization, architecture, state, threshold |
| ngspice_validation.yaml | Authored | parallel ngspice runner | Source rendering, aliases, node checks, measurements, AC/DC settings, PDK corner |

## A. SPICE Netlist Contract

The topology SPICE file remains topology-specific and parameterized. Parameter names must match source_bindings in ngspice_validation.yaml.

* structural example only
.param l_default_um=0.15
.subckt <topology> <ports...>
XM1 ... sky130_fd_pr__nfet_01v8_lvt L={l_default_um} W={w_m1_um} nf=1
XM2 ... sky130_fd_pr__pfet_01v8     L={l_default_um} W={w_m2_um} nf=1
...
.ends <topology>

Canonical geometry-unit rule. OpenAMS witness W/L values and the working SKY130 ngspice rendering path use numeric micrometers. For example W=5.125 um and L=0.15 um render as W=5.125 and L=0.15. Do not apply an extra *1e-6 conversion in source_bindings.

## B. design_rules.yaml

Carries technology/design constraints independent of a particular witness. Keep exact field names aligned with the current compiler.

schema_version: 1
technology:
  process: sky130
  corner: tt
  temperature_c: 27
  length_um: 0.15

devices:
  <device_or_group>:
    polarity: nmos|pmos
    width_um:
      min: <...>
      max: <...>

constraints:
  - <declarative sizing / matching / operating-region rule>

## C. design_intent.yaml - Hierarchy and Component-Training Metadata

The primary authored description compiled into the hierarchical component contract. It must resolve component order, feature sources/grids, interfaces, model checkpoints, and exact-realizer bindings.

schema_version: 1

hierarchical_components:
  <component_A>:
    model:
      checkpoint: technology/component_models/<component_A>.pt
      features: [<feature_1>, <feature_2>]
    training:
      features:
        <feature_1>:
          source: independent|interface:<interface_id>
        <feature_2>:
          source: independent|interface:<interface_id>
    exact_realizer:
      driver: witness_plan_builder
      coverage_bindings:
        <realizer_name>: <contract_feature_name>

interfaces:
  <interface_id>:
    variables:
      <interface_variable>:
        values: [<...>]

independent_variables:
  <independent_variable>:
    values: [<...>]

## D. design_intent.yaml - pre_spice_metrics Metadata

Topology-specific analytical device definitions and metric equations belong here, not in the generic estimator.

pre_spice_metrics:
  schema_version: 1
  technology:
    length_um: 0.15
    neighbors: 8
    nmos_model_contains: <model token>
    pmos_model_contains: <model token>

  constants:
    vdd: 1.8
    vss: 0.0
    <topology_constant>: <value>

  devices:
    M1:
      polarity: nmos
      width: w_m1_um
      vgs: <expression>
      vds: <expression>
      vbs: <expression>

  metrics:
    - name: est_gain_db
      expression: <topology-specific expression>
    - name: est_ugb_hz
      expression: <topology-specific expression>
    - name: est_power_w
      expression: <topology-specific expression>

  ranks:
    - name: rank_est_gain
      source: est_gain_db
      ascending: false

## E. hierarchical_component_contract.json - Generated Contract

Generated metadata, not the preferred hand-authored source of truth. It resolves grids, component checkpoints, interfaces, exact realizers, and preserves pre_spice_metrics.

{
  "schema_version": 3,
  "strategy": "hierarchical_component_mlp_exact_realization",
  "components": {
    "<component_id>": {
      "model": {
        "checkpoint": "technology/component_models/<model>.pt",
        "features": ["<feature_1>", "<feature_2>"]
      },
      "exact_realizer": {
        "driver": "witness_plan_builder",
        "coverage_bindings": {"<...>": "<...>"}
      }
    }
  },
  "interfaces": {"<interface_id>": {"...": "..."}},
  "independent_variables": {"<name>": {"...": "..."}},
  "pre_spice_metrics": {"...": "compiled metadata"}
}

## F. Component MLP Checkpoint Metadata

The generic trainer writes self-describing component checkpoints; the feature list and threshold must match the contract.

schema_version
kind
feature_names
mean
std
hidden
state_dict
threshold

## G. Candidate-Selection Policy

Selection objectives are currently command-line policy rather than topology YAML. The selector remains generic because it consumes arbitrary metric columns produced by pre_spice_metrics.

--count 100
--metric est_gain_db:high:0:2
--metric est_ugb_hz:high:0:2
--metric est_power_w:max:0.0005:1
--group-column independent_point_index
--max-per-group 3

Default role split:
  EXPLOIT   50%
  BOUNDARY  20%
  DIVERSITY 30%

## H. ngspice_validation.yaml - Final Validation Metadata

The complete topology-specific rendering and measurement contract for the generic ngspice stage.

schema_version: 1
name: <topology>_witness_validation
source_netlist: examples/<topology>/inputs/<topology>.spice
dc_tolerance_v: 0.05
temperature_c: 27

pdk:
  library: AUTO
  corner: tt

constants:
  vdd: 1.8
  vss: 0.0
  <simulation_constant>: <value>

source_bindings:
  l_default_um: "0.15"
  w_m1_um: w_m1_um
  w_m2_um: w_m2_um
  <other source placeholder>: <candidate column or safe expression>

witness_aliases:
  <validator_column>: <canonical_candidate_column>

circuit: |
  <supplies, input sources, DUT instance, external loads>

nodes:
  <logical_node>:
    ngspice: v(<ngspice node expression>)
    witness_column: <candidate voltage column>

measurements:
  <measurement_name>:
    ngspice: <ngspice expression>
    absolute: true

power:
  supply_voltage_v: 1.8
  current_measurement: <measurement_name>

ac:
  enabled: true
  output: v(<output>)
  sweep:
    points_per_decade: 100
    start_hz: 1.0
    stop_hz: 1.0e9
  transfer_sign: 1|-1

Validation invariants. Every nodes.*.witness_column must exist after witness_aliases are applied. Every {placeholder} in source_netlist must be resolvable by source_bindings. The selected-candidate runner simulates every supplied row and performs no second design-space selection.

## I. simulation.yaml

Where used by the topology/compiler, keep simulation environment and analysis intent here rather than in generic Python.

schema_version: 1
environment:
  temperature_c: 27
  corner: tt
  supplies:
    vdd: 1.8
    vss: 0.0

analyses:
  dc: {enabled: true}
  ac:
    enabled: true
    start_hz: 1
    stop_hz: 1e9
    points_per_decade: 100

loads:
  <load_name>: <value>

tolerances:
  <validation tolerance>: <value>

# 1. Pipeline at a Glance

design_intent.yaml
        |
        v
compile_hierarchical_component_contract.py
        |
        v
hierarchical_component_contract.json
        |
        v
generate_component_feasibility_datasets.py
        |
        +--> one dataset per component
        |
        v
train_component_model.py
        |
        +--> one .pt model per component
        |
        v
hierarchical_witness_engine.py
        |
        v
full valid circuit witness CSV

Folded-cascode L=0.15 reference flow. The current folded-cascode flow uses the ALIGN/Laygo2-compatible MOS checkpoints under technology/mlp/models_laygo2/ and the component models under technology/component_models/.

# 2. Shell Setup

Run this before any individual stage:

cd ~/AMS-Tutorial/openams
source .venv-openams/bin/activate
export PYTHONPATH="$PWD/src${PYTHONPATH:+:$PYTHONPATH}

# 3. One-Command End-to-End Pipeline

Use the generic orchestration driver when you want the complete hierarchical flow in one command.

python tools/run_generic_hierarchical_pipeline.py \
  --intent \
  examples/folded_cascode/inputs/design_intent.yaml \
  --contract \
  examples/folded_cascode/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir \
  runtime/folded_pipeline_L015 \
  --output \
  examples/folded_cascode/generated/assignment_synthesis/folded_valid_witnesses_L015.csv \
  --workers 12 \
  --clean

Generic template:

python tools/run_generic_hierarchical_pipeline.py \
  --intent examples/<topology>/inputs/design_intent.yaml \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --clean

# 4. Running a Specific Stage

The pipeline can be entered at any stage. Earlier artifacts must already exist when a preceding stage is skipped.

## Stage 1 - Compile the Hierarchical Component Contract

Use this when design_intent.yaml or the component hierarchy has changed and you need a fresh contract.

python tools/compile_hierarchical_component_contract_training.py \
  --intent examples/folded_cascode/inputs/design_intent.yaml \
  --output examples/folded_cascode/generated/assignment_synthesis/hierarchical_component_contract.json

Generic template:

python tools/compile_hierarchical_component_contract_training.py \
  --intent examples/<topology>/inputs/design_intent.yaml \
  --output examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json

## Stage 2 - Generate Component Feasibility Datasets

Use this when the contract, technology MOS MLPs, realization logic, or component sampling domains have changed.

rm -rf runtime/folded_component_training_L015

/usr/bin/time -f '\nREAL=%e sec\nMAX_RSS=%M KB' \
python tools/validation/generate_component_feasibility_datasets.py \
  --contract \
  examples/folded_cascode/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output-dir \
  runtime/folded_component_training_L015/datasets \
  --work-dir \
  runtime/folded_component_training_L015/oracle \
  --workers 12

Generic template:

python tools/validation/generate_component_feasibility_datasets.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output-dir runtime/<topology>_component_training/datasets \
  --work-dir runtime/<topology>_component_training/oracle \
  --workers 12

Generate only one component. Use the component ID exactly as it appears in the compiled contract.

python tools/validation/generate_component_feasibility_datasets.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output-dir runtime/<topology>_component_training/datasets \
  --work-dir runtime/<topology>_component_training/oracle \
  --workers 12 \
  --component <component_id>

Dry run. Use this to inspect resolved features, grids, and coverage-row counts without running the exact teacher.

python tools/validation/generate_component_feasibility_datasets.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output-dir runtime/<topology>_component_training/datasets \
  --work-dir runtime/<topology>_component_training/oracle \
  --workers 12 \
  --dry-run

## Stage 3 - Train Component Feasibility MLPs

The trainer is generic, but it is called once per component because each component has its own feature set.

mkdir -p runtime/folded_component_training_L015/models

python tools/technology/train_component_model.py \
  --dataset runtime/folded_component_training_L015/datasets/input_tail_network_dataset.csv \
  --features w_m1_um i_m3_a vp_v \
  --output runtime/folded_component_training_L015/models/folded_input_tail_network.pt

python tools/technology/train_component_model.py \
  --dataset runtime/folded_component_training_L015/datasets/upper_folded_network_dataset.csv \
  --features i_m3_a vp_v vx_v \
  --output runtime/folded_component_training_L015/models/folded_upper_folded_network.pt

python tools/technology/train_component_model.py \
  --dataset runtime/folded_component_training_L015/datasets/lower_output_network_dataset.csv \
  --features i_m3_a vx_v \
  --output runtime/folded_component_training_L015/models/folded_lower_output_network.pt

Generic per-component template:

python tools/technology/train_component_model.py \
  --dataset runtime/<topology>_component_training/datasets/<component_id>_dataset.csv \
  --features <feature_1> <feature_2> ... \
  --output runtime/<topology>_component_training/models/<component_model>.pt

The feature list should come from the component's model.features field in the hierarchical component contract. The one-command orchestration driver reads those features automatically.

## Stage 4 - Validate Component MLPs

Validation is currently topology-specific in the repository. For folded cascode, use the existing folded validator.

python tools/validation/validate_folded_component_mlp_hierarchy.py \
  --w1 5.125 \
  --i3-ua 89.875 \
  --model-a runtime/folded_component_training_L015/models/folded_input_tail_network.pt \
  --model-b runtime/folded_component_training_L015/models/folded_upper_folded_network.pt \
  --model-c runtime/folded_component_training_L015/models/folded_lower_output_network.pt \
  --work-dir runtime/folded_component_validation_L015_goodpoint

What to look for. Prioritize recall and false negatives. In the current folded test, the joined interface validation recovered all 8 exact-valid join cells (join recall 1.0) while allowing 4 extra false-positive cells. False positives cost exact-realizer runtime; false negatives can remove valid witnesses.

## Stage 5 - Install Component Models

After validation, copy the trained component checkpoints into the canonical component-model directory.

cp runtime/folded_component_training_L015/models/folded_input_tail_network.pt \
  technology/component_models/

cp runtime/folded_component_training_L015/models/folded_upper_folded_network.pt \
  technology/component_models/

cp runtime/folded_component_training_L015/models/folded_lower_output_network.pt \
  technology/component_models/

If the contract stores checkpoint paths, recompile or update the contract before the final witness run so that it points to the installed models.

## Stage 6 - Generate the Full Valid Witness Set

Use the generic hierarchical witness engine after the component MLPs are installed and the contract points to them.

rm -rf runtime/folded_generic_step5_L015

/usr/bin/time -f '\nREAL=%e sec\nMAX_RSS=%M KB' \
python tools/validation/hierarchical_witness_engine.py \
  --contract \
  examples/folded_cascode/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output \
  examples/folded_cascode/generated/assignment_synthesis/folded_valid_witnesses_L015.csv \
  --work-dir \
  runtime/folded_generic_step5_L015 \
  --workers 12

Generic template:

python tools/validation/hierarchical_witness_engine.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --work-dir runtime/<topology>_generic_step5 \
  --workers 12

# 5. Restarting the One-Command Driver from the Middle

The orchestration driver supports skip flags so that completed stages can be reused.

## Reuse an Existing Contract

python tools/run_generic_hierarchical_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --skip-contract-compile

## Reuse Existing Component Datasets and Retrain

python tools/run_generic_hierarchical_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --skip-datasets

## Reuse Existing Component Models and Only Run Final Witness Search

python tools/run_generic_hierarchical_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --skip-datasets \
  --skip-training

## Stop After Training Component Models

python tools/run_generic_hierarchical_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --work-dir runtime/<topology>_pipeline \
  --output examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --workers 12 \
  --skip-final-witnesses

# 6. Folded-Cascode Reference Results

These values are useful as a regression reference for the current L=0.15 folded-cascode flow.

| Component | Dataset Rows | Valid Rows | Positive Rate |
| --- | --- | --- | --- |
| input_tail_network | 62,775 | 4,846 | 7.720% |
| upper_folded_network | 52,731 | 2,511 | 4.762% |
| lower_output_network | 1,701 | 220 | 12.934% |

Final hierarchical witness run: 2,025 independent points; 110 points with at least one witness; 5.43% coverage; 13,422 final exact witnesses; approximately 64 s engine wall time on the recorded 12-worker run.

# 7. Technology Consistency Check for L=0.15

For the ALIGN-compatible folded flow, the transistor-level teacher must use the L=0.15 Laygo2-compatible MOS MLPs.

technology/mlp/models_laygo2/sky130_nmos_mlp.pt
technology/mlp/models_laygo2/sky130_pmos_mlp.pt

The earlier all-negative component datasets were caused by using the older technology/mlp/models/ checkpoints. Before regenerating datasets, verify that the contract/witness plan references the models_laygo2 checkpoints and length_um = 0.15.

grep -R -nE 'nmos_checkpoint|pmos_checkpoint|length_um' \
  examples/<topology>/inputs \
  examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json

# 8. Recommended Entry Point by Change Type

| What changed? | Restart from |
| --- | --- |
| design intent or component decomposition | Stage 1 - compile contract |
| MOS technology MLP or exact-realizer behavior | Stage 2 - regenerate component datasets |
| only MLP architecture/training settings | Stage 3 - retrain component MLPs |
| only model validation point/criteria | Stage 4 - validation |
| validated checkpoints only | Stage 5 - install models |
| only final hierarchy/join search settings | Stage 6 - hierarchical witness engine |

# 9. Canonical Generic Toolchain

tools/compile_hierarchical_component_contract_training.py
tools/validation/generate_component_feasibility_datasets.py
tools/technology/train_component_model.py
tools/validation/hierarchical_witness_engine.py
tools/run_generic_hierarchical_pipeline.py

Current non-generic validation helpers: validate_folded_component_mlp_hierarchy.py and validate_two_stage_component_mlp_hierarchy.py. These are useful regression tools but are not part of the generic synthesis core.

# 10. Generic Pre-SPICE Candidate-Selection Stage

Purpose. After the hierarchical witness engine produces an exact valid-witness CSV, this stage estimates inexpensive DC/AC ranking metrics for every witness and selects a compact, diverse set of candidates for ngspice validation. The metric engine and selector are topology-agnostic; topology-specific metric definitions are stored declaratively in design_intent.yaml and preserved in the compiled contract.

valid_witnesses.csv
        |
        v
compiled contract: pre_spice_metrics metadata
        |
        v
estimate_pre_spice_metrics.py
        |
        v
all witnesses + estimated metrics
        |
        v
select_pre_spice_candidates.py
        |
        +--> EXPLOIT candidates
        +--> BOUNDARY candidates
        +--> DIVERSITY candidates
        |
        v
ngspice_candidates.csv

## 10.1 Metric Metadata and Contract Compilation

Architecture rule. The generic estimator contains no transistor names, fixed device count, or circuit-specific gain equations. Device operating-point expressions, technology length, constants, and first-order metric equations are declared under pre_spice_metrics in design_intent.yaml. The compiler preserves this metadata in hierarchical_component_contract.json.

Compile or refresh the contract with metric metadata:

python tools/compile_hierarchical_component_contract_metrics.py \
  --intent examples/<topology>/inputs/design_intent.yaml \
  --output examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json

Folded-cascode metadata preparation:

python tools/upgrade_folded_pre_spice_metadata.py \
  --intent examples/folded_cascode/inputs/design_intent.yaml

Two-stage op-amp metadata preparation:

python tools/upgrade_two_stage_pre_spice_metadata.py \
  --intent examples/two_stage_opamp/inputs/design_intent.yaml

## 10.2 One-Command Pre-SPICE Pipeline

The canonical entry point for this stage is run_pre_spice_candidate_pipeline.py. It first runs generic metric estimation and then generic performance/diversity candidate selection.

Generic template:

python tools/run_pre_spice_candidate_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --technology-csv technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output examples/<topology>/generated/assignment_synthesis/pre_spice_metrics.csv \
  --candidates-output examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

## 10.3 Folded-Cascode Example

python tools/run_pre_spice_candidate_pipeline.py \
  --contract \
  examples/folded_cascode/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses \
  examples/folded_cascode/generated/assignment_synthesis/folded_valid_witnesses_L015_v2.csv \
  --technology-csv \
  technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output \
  examples/folded_cascode/generated/assignment_synthesis/folded_pre_spice_metrics_L015_v2.csv \
  --candidates-output \
  examples/folded_cascode/generated/assignment_synthesis/folded_ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

Regression-qualified folded result. 13,422 exact witnesses were processed. The selector found 10,820 rows meeting the hard estimated power constraint and selected 100 candidates: 50 EXPLOIT, 20 BOUNDARY, and 30 DIVERSITY. The selected set represented 43 independent design points.

## 10.4 Two-Stage Op-Amp Example

python tools/run_pre_spice_candidate_pipeline.py \
  --contract \
  examples/two_stage_opamp/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses \
  examples/two_stage_opamp/generated/assignment_synthesis/two_stage_valid_witnesses_L015.csv \
  --technology-csv \
  technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output \
  examples/two_stage_opamp/generated/assignment_synthesis/two_stage_pre_spice_metrics_L015.csv \
  --candidates-output \
  examples/two_stage_opamp/generated/assignment_synthesis/two_stage_ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

Regression-qualified two-stage result. 302,767 exact witnesses were processed. The selector found 258,622 rows meeting the hard estimated power constraint and selected 100 candidates: 50 EXPLOIT, 20 BOUNDARY, and 30 DIVERSITY. The selected set represented 60 independent design points. After the diversity-selection scalability fix, the 302,767-row selection completed in approximately 12.4 seconds.

## 10.5 Candidate-Selection Policy

The default 100-point selection budget is deliberately mixed so ngspice does not see only near-duplicate high-scoring designs.

| Role | Default Share | Purpose |
| --- | --- | --- |
| EXPLOIT | 50% | Best estimated metric/spec score. |
| BOUNDARY | 20% | Near hard estimated specification boundaries; informative for calibration. |
| DIVERSITY | 30% | Geometrically separated device-size combinations. |

Metric syntax:

--metric COLUMN:DIRECTION:LIMIT[:WEIGHT[:SCALE]]

Directions:
  min     value should be >= LIMIT
  max     value should be <= LIMIT
  target  value should be near LIMIT
  high    larger is better; soft ranking objective
  low     smaller is better; soft ranking objective

Current recommended first-pass policy. Treat analytical gain and UGB estimates as ranking objectives (high), not hard sign-off thresholds, until correlation with actual ngspice results has been measured. Power can be used as a hard pre-SPICE filter.

## 10.6 Running Individual Steps

Metric estimation only:

python tools/validation/estimate_pre_spice_metrics.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --technology-csv technology/sky130_tt_27c_mos_characterization.csv \
  --output examples/<topology>/generated/assignment_synthesis/pre_spice_metrics.csv

Candidate selection only:

python tools/validation/select_pre_spice_candidates.py \
  --input examples/<topology>/generated/assignment_synthesis/pre_spice_metrics.csv \
  --output examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3

Reuse an existing metrics CSV and rerun selection through the one-command driver:

python tools/run_pre_spice_candidate_pipeline.py \
  --contract examples/<topology>/generated/assignment_synthesis/hierarchical_component_contract.json \
  --witnesses examples/<topology>/generated/assignment_synthesis/valid_witnesses.csv \
  --technology-csv technology/sky130_tt_27c_mos_characterization.csv \
  --metrics-output examples/<topology>/generated/assignment_synthesis/pre_spice_metrics.csv \
  --candidates-output examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --count 100 \
  --metric est_gain_db:high:0:2 \
  --metric est_ugb_hz:high:0:2 \
  --metric est_power_w:max:0.0005:1 \
  --group-column independent_point_index \
  --max-per-group 3 \
  --skip-metrics

# 11. Updated End-to-End OpenAMS Flow Through Parallel ngspice Validation

INPUT FILES
  SPICE netlist
  design_rules.yaml
  design_intent.yaml
  simulation.yaml
  ngspice_validation.yaml
        |
        v
run_generic_hierarchical_pipeline.py
        |
        v
valid_witnesses.csv
        |
        v
run_pre_spice_candidate_pipeline.py
        |
        +--> pre_spice_metrics.csv
        |
        v
ngspice_candidates.csv
        |
        v
run_ngspice_candidates_parallel.py
        |
        +--> ngspice_validation.csv
        +--> validated_candidates.csv
        +--> run_manifest.json

# 12. Updated Canonical Generic Toolchain

tools/compile_hierarchical_component_contract_training.py
tools/compile_hierarchical_component_contract_metrics.py
tools/validation/generate_component_feasibility_datasets.py
tools/technology/train_component_model.py
tools/validation/hierarchical_witness_engine.py
tools/run_generic_hierarchical_pipeline.py

tools/validation/estimate_pre_spice_metrics.py
tools/validation/select_pre_spice_candidates.py
tools/run_pre_spice_candidate_pipeline.py
tools/validation/run_ngspice_candidates_parallel.py
src/openams/validation/ngspice_witness.py

Current topology-specific metadata preparation helpers: upgrade_folded_pre_spice_metadata.py and upgrade_two_stage_pre_spice_metadata.py. These populate declarative topology metadata; the estimator and candidate-selection algorithms remain generic.

# 13. Generic Parallel ngspice Validation Stage

Purpose. This final stage takes an already-selected candidate CSV and simulates every row. The generic runner prepares validator-local identity, applies declarative witness aliases, preflights required witness columns, shards deterministically, launches ngspice workers, and merges measured results back to the selected candidate table.

python tools/validation/run_ngspice_candidates_parallel.py \
  --plan examples/<topology>/inputs/ngspice_validation.yaml \
  --candidates examples/<topology>/generated/assignment_synthesis/ngspice_candidates_100.csv \
  --root "$PWD" \
  --workers 12 \
  --output-dir runtime/ngspice_<topology>_selected_100 \
  --overwrite

Primary outputs are ngspice_validation.csv, validated_candidates.csv, prepared_candidates.csv, worker_manifest.csv, and run_manifest.json.

Regression status. The latest generic runner has been exercised on both the two-stage op amp and folded cascode. The two-stage final regression reproduced 54/100 DC/total PASS and the same AC distribution as the earlier run. The folded cascode completed all 100 ngspice simulations after its topology metadata was corrected to the canonical micrometer W/L rendering convention.